Causal Phase Recognition for Wall-Normal Shared Control in Remote Robotic Painting

- Introduction

- Motivation

  - Observation from the preceding remote-painting experiment

    - Persistent difficulty maintaining detailed EOAT–wall contact during teleoperation

    - Concurrent operator demand: tangential path control and wall-normal contact correction

  - Shared-control opportunity

    - Operator authority over tangential painting motion

    - Robot assistance for the wall-normal component and contact regulation

- Need for causal phase recognition

  - Phase-dependent assistance logic

    - FREE MOTION: assistance disengagement

    - APPROACH: safe convergence without premature contact locking

    - PAINTING: wall-normal regulation during surface interaction

    - RETRACT: prompt assistance release without resisting withdrawal

  - No fixed phase order; no imposed minimum phase duration

  - Operational risk from delayed, missed, or spurious phase transitions

- Research problem

  - Rule-based logic versus data-driven causal classifiers under a common input and replay protocol

  - Dominant PAINTING class versus brief APPROACH and RETRACT intervals

  - Generalization across training-set composition, trials, and participants

- Study aim

  - Comparative evaluation of deterministic, framewise, probabilistic sequential, and recurrent models

  - Effect of training-data structure and scale on model ranking

  - Learned spatial memory as a surface-relative cue without an explicit wall rule

- Research questions

  - RQ1. Which causal model offers the most useful trade-off among phase F1, transition behavior, and interpretability?

  - RQ2. How do training-set composition and scale alter held-out performance?

  - RQ3. Does a learned position-memory gate improve minority-phase and transition recognition?

- Study Data and Phase Definition

- Source dataset

  - Preceding remote robotic painting experiment

  - Six participants; four visual-interface conditions; 22 trials; 61 continuous sequences

  - 208,430 labeled frames; 207,488 frames with valid causal features; 115.8 min

- Causal input representation

  - 12 dimensions per frame

    - TCP force: x, y, z

    - Position relative to trial start: x, y, z

    - Backward-difference velocity: x, y, z

    - Backward-difference acceleration: x, y, z

  - Training-set standardization only; no future-frame features

- Manual phase labels

  - FREE MOTION, APPROACH, PAINTING, RETRACT

  - Dataset distribution

    - FREE MOTION: 29.6%; APPROACH: 5.0%; PAINTING: 62.7%; RETRACT: 2.8%

  - Minority-phase emphasis: APPROACH and RETRACT

- Causal Phase-Recognition Methods

- Causal FSM

  - Training-calibrated force and motion thresholds; deterministic online state logic

- Random Forest

  - Nonlinear framewise tree ensemble; independent prediction per observation

- Gaussian Mixture Model

  - Phase-conditional mixture likelihoods; framewise maximum-posterior prediction

- Gaussian HMM

  - Single-Gaussian state emissions; causal forward filtering

- GMM-HMM

  - Mixture emissions with learned state transitions; causal forward filtering

- Causal LSTM

  - One-layer recurrent classifier; 64 hidden units

  - 120-frame training chunks; batch size 32; 20 epochs; learning rate 0.001

  - Unweighted cross-entropy; persistent hidden state during one-frame online replay

- Causal LSTM with learned spatial memory

  - Original 12D observation xₜ retained

  - Learned scalar gate gₜ: 0 = retain memory; 1 = write current position

  - Position memory: mₜ = (1 − gₜ)mₜ₋₁ + gₜpₜ

  - Internal relative position: rₜ = pₜ − mₜ

  - LSTM input: [xₜ, rₜ, gₜ]

  - Phase cross-entropy only; no wall coordinates, gate labels, or gate regularizer

- Evaluation Protocol

- Training-structure comparison; common held-out trial 08_01

  - Participant-specific: 10_01–10_04; four trials

  - Single-interface: 06_01, 07_01, 10_01, 11_01; four trials

  - Diversity-balanced: 06_03, 09_04, 10_01, 11_02; four trials

  - All available training data: 21 trials; 08_01 excluded

- Participant-independent evaluation

  - Leave-one-participant-out; six held-out participant folds

- Online replay

  - One observation per model step; temporal state reset only at sequence boundaries

  - No future frames; no test-trial scaler fitting; no test-trial model fitting

- Metrics

  - Frame classification: accuracy, balanced accuracy, Macro F1, per-phase F1

  - Transition detection: matched recall and F1 within ±2.0 s

  - Operational stability: false transitions, absolute transition delay, inference time

  - Transition recall: proportion of true phase changes matched by a prediction; not overall frame accuracy

- Spatial-memory effect verification

  - Baseline 12D LSTM, DSA 13D representation, and full learned-memory model

  - Existing diagnostics: capacity-matched plain LSTM, masked inputs, and retrained partial-input variants

- Results

- Training-data structure and scale (Table 1; Figures 1–5)

  - With four training trials, model ranking depended on how the trials were selected; no four-trial structure dominated every model.

  - Using all 21 training trials improved Macro F1 for every learned model and moved the Causal LSTM to the strongest overall held-out result.

  - Random Forest remained highly competitive in low-data settings, indicating a lower sample requirement than the recurrent model.

Table 1

Held-Out Classification Performance by Training-Set Structure

| Model | P10-4 | I01-4 | DIV-4 | ALL-21 |
| --- | --- | --- | --- | --- |
| Causal FSM | 61.8 / 25.1 | 42.3 / 26.2 | 49.9 / 23.6 | 54.9 / 24.5 |
| Random Forest | 83.5 / 48.0 | 84.2 / 54.5 | 81.9 / 41.8 | 87.9 / 61.7 |
| GMM | 76.6 / 39.7 | 80.1 / 46.8 | 79.4 / 48.6 | 82.3 / 54.2 |
| Gaussian HMM | 79.0 / 42.6 | 81.1 / 48.5 | 75.8 / 42.4 | 77.3 / 51.7 |
| GMM-HMM | 75.5 / 39.8 | 74.8 / 45.1 | 76.4 / 46.3 | 80.4 / 50.9 |
| Causal LSTM | 78.2 / 50.1 | 82.0 / 54.7 | 75.9 / 46.0 | 89.3 / 64.1 |

Note. Values are Accuracy / Macro F1 in percentages. All models were evaluated on the same unseen trial, 08_01. P10-4 = four trials from participant 10; I01-4 = four trials from interface condition 01; DIV-4 = four condition-diverse trials; ALL-21 = all remaining 21 trials.

Figure 1

Macro F1 Across Four Training-Set Structures

Note. The 21-trial condition yielded the highest Macro F1 for each learned model. The rule-based FSM did not show the same scale-related gain.

Figure 2

Whole-Trial Phase Labels for Participant-specific, 4 trials

Note. Ground truth and one-frame causal predictions on 08_01. Training set: 10_01–10_04; 27,813 training frames. Gray intervals indicate timestamp gaps or feature warm-up intervals.

Figure 3

Whole-Trial Phase Labels for Single-interface, 4 trials

Note. Ground truth and one-frame causal predictions on 08_01. Training set: 06_01, 07_01, 10_01, 11_01; 33,251 training frames. Gray intervals indicate timestamp gaps or feature warm-up intervals.

Figure 4

Whole-Trial Phase Labels for Diversity-balanced, 4 trials

Note. Ground truth and one-frame causal predictions on 08_01. Training set: 06_03, 09_04, 10_01, 11_02; 27,451 training frames. Gray intervals indicate timestamp gaps or feature warm-up intervals.

Figure 5

Whole-Trial Phase Labels for All available training data, 21 trials

Note. Ground truth and one-frame causal predictions on 08_01. Training set: all trials except 08_01; 193,014 training frames. Gray intervals indicate timestamp gaps or feature warm-up intervals.

- Detailed 21-trial model comparison (Tables 2–3)

  - Causal LSTM provided the strongest joint classification result and the fewest spurious transitions, while Random Forest retained the highest RETRACT F1.

  - GMM detected the greatest proportion of true transitions but produced many additional false transitions; transition sensitivity alone did not indicate reliable phase tracking.

Table 2

Per-Phase Performance After Training on 21 Trials

| Model | Accuracy | Macro F1 | FREE F1 | APP. F1 | PAINT. F1 | RET. F1 |
| --- | --- | --- | --- | --- | --- | --- |
| Causal FSM | 54.9 | 24.5 | 21.0 | 0.0 | 72.4 | 4.5 |
| Random Forest | 87.9 | 61.7 | 73.2 | 39.8 | 92.9 | 40.8 |
| GMM | 82.3 | 54.2 | 67.0 | 25.6 | 90.0 | 34.4 |
| Gaussian HMM | 77.3 | 51.7 | 68.9 | 20.5 | 85.4 | 32.2 |
| GMM-HMM | 80.4 | 50.9 | 68.2 | 21.5 | 89.8 | 24.2 |
| Causal LSTM | 89.3 | 64.1 | 76.8 | 49.7 | 94.0 | 35.8 |

Note. Values are percentages on unseen trial 08_01. APP. = APPROACH; PAINT. = PAINTING; RET. = RETRACT. Test distribution: FREE 2,861 frames; APPROACH 336; PAINTING 11,078; RETRACT 199.

Table 3

Transition Behavior After Training on 21 Trials

| Model | Transition recall | Transition F1 | False transitions | ¦Delay¦, ms |
| --- | --- | --- | --- | --- |
| Causal FSM | 9.8 | 1.6 | 468 | 458 |
| Random Forest | 68.3 | 15.9 | 283 | 303 |
| GMM | 82.9 | 9.5 | 644 | 377 |
| Gaussian HMM | 68.3 | 20.8 | 200 | 360 |
| GMM-HMM | 63.4 | 13.2 | 326 | 401 |
| Causal LSTM | 58.5 | 40.3 | 54 | 401 |

Note. Transitions were matched by phase-pair identity and nearest unused prediction within ±2.0 s. Transition recall is the proportion of 41 true transitions matched; false transitions are unmatched predicted changes.

- Participant-independent generalization (Table 4; Figure 6)

  - Across unseen participants, the Causal LSTM retained the strongest mean accuracy and Macro F1, but participant-level variance remained substantial.

  - Probabilistic sequence models showed temporal smoothing benefits without matching the recurrent model's aggregate classification performance.

Table 4

Leave-One-Participant-Out Performance

| Model | Accuracy | Macro F1 | APP. F1 | RET. F1 | Transition recall |
| --- | --- | --- | --- | --- | --- |
| Causal FSM | 60.5 (11.6) | 32.3 (5.1) | 9.5 (9.7) | 8.9 (4.7) | 13.6 (5.4) |
| Random Forest | 78.7 (8.3) | 52.1 (10.4) | 31.7 (18.9) | 22.4 (12.7) | 60.2 (13.0) |
| GMM | 74.2 (9.2) | 51.1 (6.9) | 32.3 (14.3) | 21.1 (9.7) | 74.7 (3.9) |
| Gaussian HMM | 69.5 (9.7) | 48.9 (4.3) | 28.8 (7.1) | 24.4 (9.6) | 56.4 (7.8) |
| GMM-HMM | 69.4 (10.8) | 49.2 (5.9) | 32.8 (14.0) | 18.0 (6.8) | 58.4 (8.9) |
| Causal LSTM | 80.3 (7.3) | 59.7 (8.4) | 49.4 (14.8) | 31.8 (12.6) | 61.7 (16.0) |

Note. Values are mean (SD) percentages across six participant folds. Each fold trained on all other participants and replayed the held-out participant causally, one frame at a time.

Figure 6

Participant-Independent Macro F1

Note. Bars show fold means; error bars show one standard deviation across held-out participants.

- Spatial-memory extension (Tables 5–6; Figures 7–8)

  - The full spatial-memory LSTM improved balanced accuracy, Macro F1, RETRACT F1, and transition recall over the baseline LSTM, while preserving the strong PAINTING result.

  - The learned gate was generally high during PAINTING and lower during FREE MOTION and APPROACH, consistent with continual updating of a local surface memory rather than storing only the first contact point.

  - Capacity control and partial-input diagnostics indicated that the gain was not explained by parameter count alone and that the full interaction among xₜ, rₜ, and gₜ was important.

Table 5

LSTM Input and Spatial-Memory Comparison

| Model | Accuracy | Balanced | Macro F1 | APP. F1 | RET. F1 | Trans. recall | Trans. F1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline LSTM (12D) | 89.3 | 59.7 | 64.1 | 49.7 | 35.8 | 58.5 | 40.3 |
| DSA LSTM (13D) | 88.0 | 61.5 | 62.8 | 46.0 | 34.3 | 58.5 | 35.8 |
| Spatial-memory LSTM | 90.1 | 63.3 | 66.1 | 50.0 | 38.9 | 70.7 | 43.6 |

Note. Values are percentages on unseen trial 08_01. Baseline = original 12D input; DSA = distance, speed, and acceleration representation; spatial memory = original 12D input plus learned rₜ and gₜ.

Table 6

Component-Effect Verification for the Spatial-Memory LSTM

| Condition | Accuracy | Balanced | Macro F1 | APP. F1 | RET. F1 | Trans. F1 | Parameters |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Baseline LSTM, h=64 | 89.3 | 59.7 | 64.1 | 49.7 | 35.8 | 40.3 | 20,228 |
| Capacity-matched LSTM, h=74 | 89.7 | 56.7 | 60.8 | 37.6 | 32.6 | 38.5 | 26,348 |
| Full spatial model, x+r+g | 90.1 | 63.3 | 66.1 | 50.0 | 38.9 | 43.6 | 26,437 |
| Full weights; r masked | 89.6 | 57.6 | 61.0 | 31.0 | 38.9 | 41.5 | 26,437 |
| Full weights; g masked | 81.7 | 62.3 | 59.3 | 50.0 | 29.0 | 25.7 | 26,437 |
| Retrained x+r | 88.9 | 59.3 | 63.4 | 45.1 | 37.8 | 35.1 | 26,181 |
| Retrained x+g | 88.6 | 59.9 | 63.1 | 47.9 | 33.5 | 39.7 | 25,669 |

Note. Values are percentages except Parameters. Input masking used the trained full-model weights and is diagnostic rather than a separately optimized model. Retrained partial-input variants used the same held-out trial. h = hidden units.

Figure 7

Performance of the Baseline, DSA, and Learned Spatial-Memory LSTMs

Note. The full spatial-memory model produced the strongest balanced, Macro F1, RETRACT F1, and transition F1 values among the three LSTM representations.

Figure 8

Learned Gate and Relative-Position Memory During Causal Replay

Note. The gate controls memory writing, not the PAINTING label directly. A high gate repeatedly moves memory toward the current position; the resulting rₜ measures local displacement from that learned reference. The trace supports functional use but does not establish a geometrically exact wall model.

- Discussion

- Model trade-offs

  - Causal FSM

    - Strength: transparent logic, negligible training demand, direct safety review

    - Weakness: brittle thresholds, poor transient-phase recognition, heavy calibration dependence

  - Random Forest

    - Strength: strong low-data baseline, nonlinear decision boundaries, simple fitting

    - Weakness: no temporal memory, greater susceptibility to framewise chattering and false transitions

  - GMM

    - Strength: high transition sensitivity, compact probabilistic phase representation

    - Weakness: phase-overlap ambiguity and many spurious transitions without temporal state

  - Gaussian HMM

    - Strength: causal temporal smoothing and interpretable state transitions

    - Weakness: rigid single-Gaussian emissions and delayed or weakened brief-phase responses

  - GMM-HMM

    - Strength: multimodal emissions with temporal continuity

    - Weakness: added complexity without top-ranked performance; dependence on learned transition statistics

  - Causal LSTM

    - Strength: strongest data-rich and participant-independent aggregate performance; few false transitions

    - Weakness: greater data requirement, weaker interpretability, remaining transition-recall and delay trade-off

  - Spatial-memory LSTM

    - Strength: improved minority-phase balance and transition detection through a learned local spatial reference

    - Weakness: modest single-trial gain, indirect gate semantics, possible memory drift across unfamiliar geometry

- Interpretation of training-scale effects

  - Tree ensemble advantage under limited trials: strong framewise pattern fitting with low sequence-data demand

  - LSTM advantage under expanded trials: improved learning of temporal variability and participant-specific motion patterns

  - Implication: model ranking inseparable from available sequence diversity and evaluation split

- Meaning of learned spatial memory

  - End-to-end emergence of a task-useful surface-relative latent cue from phase labels alone

  - Continuous PAINTING-period updates consistent with a local wall manifold, not a single memorized contact point

  - Gate value as memory-write strength; not an independent PAINTING detector

  - Evidence for integrating structured inductive bias without explicit wall coordinates or phase-order rules

- Limitations

  - Single wall geometry, EOAT, task family, and laboratory context

  - Only six participants; one common held-out trial for training-structure and spatial-memory comparisons

  - Severe class imbalance and short APPROACH/RETRACT intervals

  - Transition metric dependence on a ±2.0-s matching window

  - No closed-loop user evaluation of phase-triggered wall-normal shared control

  - No geometric ground truth proving that mₜ equals the physical wall surface

- Next validation

  - Closed-loop shared-control experiment: safety, contact stability, operator workload, and task quality

  - Cross-geometry and cross-tool validation; uncertainty-aware assistance release

  - Gate and memory diagnostics against measured surface geometry

- Contributions

- Application-driven formulation of causal phase recognition for wall-normal shared control in remote robotic painting

- Common one-frame causal benchmark spanning rules, framewise learning, probabilistic sequence models, and recurrent learning

- Empirical evidence that training-set scale and composition can change the preferred model

- Learned spatial-memory LSTM requiring neither explicit wall coordinates nor fixed phase-order and duration constraints

- Evaluation emphasis on minority-phase F1, transition matching, false transitions, and delay beyond overall accuracy